## 1. SMM-dx Project 1 – Universal Fukiage Detection (Copper Surface)

### Objective

Develop a camera-agnostic algorithm to detect and quantify Fukiage (flash/flare) on copper surfaces, robust to:

- Partial object visibility

- Different camera FOVs

- Illumination extremes

### Algorithm Pipeline

#### 1.1 Brightness Validation

- Convert input image to grayscale.

- Compute global statistics (mean, max intensity).

- Decision logic:

  - Overexposed → return -1

  - Underexposed → return 0

  - Valid → proceed to detection

This prevents false Fukiage estimation under invalid lighting.

#### 1.2 Universal Contour Handling

- Binary segmentation using adaptive/OTSU thresholding.

- Extract largest contour.

- Fit minimum-area rotated rectangle.

- Completeness check:

  - If rectangle width < expected → full contour.

  - Else → object touches image boundaries.

Partial-object correction:

- Use image borders to reconstruct missing contour segments.

- Form a closed polygon approximating the full copper surface.

- Enables correct area estimation for Camera-4 images.

#### 1.3 Difference-Based Fukiage Isolation

- Create row-shifted and column-shifted images.

- Compute absolute differences to amplify high-frequency intensity changes:

D=∣I−Irow∣+∣I−Icol∣D = |I - I_{row}| + |I - I_{col}|D=∣I−Irow​∣+∣I−Icol​∣

- This suppresses smooth copper texture and highlights flare boundaries.

#### 1.4 Morphological Cleanup

- Erode reconstructed mask to remove boundary noise.

- Apply mask to difference image.

#### 1.5 Quantification

Fukiage %=Fukiage PixelsCopper Area×100\text{Fukiage \%} = \frac{\text{Fukiage Pixels}}{\text{Copper Area}} \times 100Fukiage %=Copper AreaFukiage Pixels​×100

Final output is consistent across camera setups.

## 2. Noritake Retail – Fabric Scratch Detection

### Problem

Detect faint scratches on knitted fabric while ignoring:

- Fabric texture

- Shadows

- Lighting variation

### Algorithm Design

#### 2.1 Dynamic Shadow Region Detection

- Compute column-wise mean intensity.

- Smooth using moving average.

- Locate peak (optimal lighting).

- Select central window (~40% width).

- Adaptive thresholding to detect shadow zones.

- Extract polygon using contour + rotated rectangle expansion.

#### 2.2 Histogram-Driven Thresholding

- Compute x-axis intensity histogram.

- Smooth histogram.

- Detect sharp gradient changes.

- Dynamically infer lower/upper thresholds.

This replaces brittle hardcoded thresholds.

#### 2.3 High-Density Scratch Enhancement

- Divide image into 50×50 blocks.

- Replace each block with its mean → smooth illumination image.

- Subtract smooth image from original:

Ihf=I−IsmoothI_{hf} = I - I_{smooth}Ihf​=I−Ismooth​

- Threshold dense regions to isolate scratches.

- Fully vectorized NumPy implementation for speed.

## 3. Mitsubishi Gas Chemical – Fine Surface Scratch Detection

### Challenge

Detect fine scratches under non-uniform lighting with minimal over-detection.

### Difference-from-Average Method

- Convert image to grayscale.

- Generate:

  - Gaussian blurred image

  - Block-average image (50×50)

- Compute:

D=∣Igaussian−Iaverage∣D = |I_{gaussian} - I_{average}|D=∣Igaussian​−Iaverage​∣

- Threshold to highlight candidate scratches.

- Extract high-density regions.

- Final thresholding to suppress noise.

- Contour extraction and annotation.

This normalizes illumination while preserving fine surface defects.

## 4. Teeth Anomaly Detection – Rotary Machinery

### Core Issue

Camera vibration makes static ROI-based detection unreliable.

### 4.1 Motion Stabilization

- Detect ORB keypoints per frame.

- Track keypoints using Lucas–Kanade Optical Flow.

- Compute displacement vectors.

- Use median displacement to estimate camera motion:

(Δx,Δy)=median(Δxi,Δyi)(\Delta x, \Delta y) = \text{median}(\Delta x_i, \Delta y_i)(Δx,Δy)=median(Δxi​,Δyi​)

- Shift bounding rectangle accordingly.

This ensures consistent teeth alignment across frames.

### 4.2 Anomaly Detection

- Train Detectron2 to segment teeth region.

- Mask non-teeth areas.

- Train PaDiM with Wide-ResNet-50 on “good” samples.

- Use Mahalanobis distance to detect deviations.

## 5. Amano Enzyme – Tank Level Estimation & Camera Drift

### 5.1 Height Estimation

#### Preprocessing

- Resize images to 224×224.

- Extract timestamps from filenames.

- Enable time-series ordering.

#### Time-Series Interpolation

- Use scipy.interpolate.interp1d to handle irregular sampling.

#### Regression Model

- Backbone: ResNet-50 (frozen).

- Head: GAP → FC → ReLU → Dropout → Sigmoid.

- Optimizer: Adam.

- Loss: MAE.

### 5.2 Camera Angle Change Detection

- Standardize features.

- Apply PCA for noise reduction.

- Compute Mahalanobis distance in PCA space.

- Trigger alert when distance exceeds threshold.

## 6. SMM Nickel – Multi-Modal Anomaly Detection

### Three-Layer Detection Architecture

#### 6.1 Visible Anomalies

- Mask R-CNN (Detectron2).

- Segments nickel region and visible foreign objects.

#### 6.2 Non-Visible Texture Anomalies

- Difference of Gaussian (DoG) filtering.

- Row/column shifting to amplify edges.

- Mask nickel region.

- Contour-based candidate extraction.

#### 6.3 Color Anomalies

- Convert to LAB color space.

- Model normal color distribution (mean, covariance).

- Segment using superpixels.

- Compute Mahalanobis distance:

DM=(x−μ)TΣ−1(x−μ)D_M = \sqrt{(x-\mu)^T \Sigma^{-1} (x-\mu)}DM​=(x−μ)TΣ−1(x−μ)​

- Threshold to detect rust/stains.

Final anomaly mask = union of all layers.

## 7. Kikkoman – Bottle Scratch Detection

### 7.1 View-Based Categorization

- Extract ResNet-50 features.

- Apply K-Means clustering to group camera angles.

- Train separate models per view.

### 7.2 Synthetic Defect Generation

- Inject geometric defects (lines, circles, polygons).

- Size constrained by real-world measurements.

- Generates accurate ground-truth masks.

### 7.3 Anomaly Detection & Deployment

- Train PaDiM per view.

- Export models to OpenVINO IR.

- Enables high-throughput CPU inference.

## 8. Nippon Signal – Bolt Presence Detection

### Pipeline

- Dataset split: train / validation / test.

- Manual bolt annotation (COCO format).

- Data augmentation:

  - Brightness

  - Contrast

  - Noise

- Fine-tune Mask R-CNN (Detectron2).

- Evaluate on held-out test set.

Produces reliable bolt presence detection under varied conditions.

## 9. Paloma Rheem Holdings – Robotic Manipulation

### 9.1 Data Collection

- Integrate DOBOT Nova-5 and Inspire Hands SDKs.

- ROS2 node records:

  - Images

  - Robot pose

  - Joint states

  - Actions

### 9.2 Dataset Preparation

- Convert episodes to RLDS format.

- Ensure compatibility with Octo framework.

### 9.3 Policy Learning

- Fine-tune pretrained Octo transformer model.

- Visual + proprioceptive inputs → action outputs.

- Supports both simulation and real robot inference.

## 10. Azusa Hinohara – Drone Pathfinding (from earlier report)

### Modified Dijkstra Algorithm

- Standard shortest-path search.

- Add slope constraint:

Slope=arctan⁡(hB−hAd)\text{Slope} = \arctan\left(\frac{h_B - h_A}{d}\right)Slope=arctan(dhB​−hA​​)

- Valid move only if slope < 30°.

- Prioritized neighbor expansion by distance.

Ensures physically feasible drone navigation.